PEGN 513-A: Reservoir Simulation I

October 13, 2009

#1: Flow velocities. (20 points)

- The velocity vector in 2-D is: . Assuming that x and y are not the principal permeability directions, write down the expressions for the Darcy velocity components,, in terms of the appropriate permeability tensor components. Indicate units for a set of consistent units, such as the field units of ft/day, md, cp, psi, etc.

- Suppose x and y are the principal permeability directions with permeability values equal to 100 md and 10 md, respectively. Write down the velocity vector component for this case with appropriate units.

- Define interstitial total velocity.

- Define frontal velocity for water displacing oil.

- Could you define interstitial phase velocity for the water phase?

#2: Water displacing oil. (30 points)

- Write down the mass balance partial differential equation for water and oil assuming no interphase mass transfer.

- Assuming constant, but small, compressibilities for water and oil derive the two-phase pressure equation using the mass balance approach in Part (a).

- What is the expression for the total velocity vector for water-oil flow?

- Derive the 2-D analog of the Buckley-Leverett equation using the water-oil total velocity and the water phase mass balance equation.

- Derive the classical 1-D Buckley-Leverett equation from the 2-D equation above.

#3: Numerical solutions. (30 points)

- Define the viscous, gravity, capillary and compressive components of the flow in the 2-D equation of Problem #2.

- Set up an explicit finite-difference equation to solve the waterflood in 1-D. Show your formulation for a 5-node grid.

- Define the stability criterion for the explicit upstream-weighted formulation.

- Define the numerical dispersion for both the upstream-weighted explicit and implicit formulations. Give units for all variables.

- Define capillary dispersion. How would you estimate its magnitude and give units for all variables.

#4: Relative permeability and capillary pressure. (20 points)

- Plot schematics of capillary pressure and relative permeability curves used in waterflooding simulation or calculations.

- What is fraction flow in terms of viscous, gravity, and capillary forces?