PEGN 513

Reservoir Simulation I

Fall 2009

Homework # 11

Due Date: 11/17/2009

Luis E. Zerpa

Homework #11

a) Implement project option 1 in 1D.

The IMPES formulation was implemented and coded in a Matlab script, shown in Addendum A.1, using the formulation presented in the HW #10 report.

b) Implement project option 2 in 1D.

The semi-implicit formulation was implemented and coded in a Matlab script, shown in Addendum A.2, using the formulation presented in the HW #10 report.

In this work the pressure and saturation equations linear systems were solved using the Thomas algorithm for tri-diagonal matrices, based on the LU decomposition of the matrix and taking advantaged of the matrix structure.

c) Results of the simulation

IMPES formulation results

Figure 1, shows a plot of oil pressure as a function of distance along path of flow at every five time steps, obtained using the IMPES formulation.

Figure 1. Oil pressure as a function of distance along path of flow at every five time steps (IMPES formulation)

Figure 2, shows a plot of the water saturation profile as a function of distance along path of flow at every five time steps, obtained using the IMPES formulation.

Figure 2. Water saturation profile as a function of x coordinate, for every five time steps (IMPES formulation)

Figure 3, shows a plot of the recovery factor as a function of time, obtained with the IMPES formulation. The total recovery factor at 10 days is 51.4515 % of OOIP.

Figure 3. Recovery factor as a function of time (IMPES formulation)

Semi-implicit formulation results

Figure 4, shows a plot of oil pressure as a function of distance along path of flow at every five time steps, obtained using the Semi-implicit formulation.

Figure 4. Oil pressure as a function of distance along path of flow at every five time steps (Semi-implicit formulation)

Figure 5, shows a plot of the water saturation profile as a function of distance along path of flow at every five time steps, obtained using the IMPES formulation.

Figure 5. Water saturation profile as a function of x coordinate, for every five time steps (Semi-implicit formulation)

Figure 6, shows a plot of the recovery factor as a function of time, obtained with the Semi-implicit formulation. The total recovery factor at 10 days is 51.4499 % OOIP.

Figure 6. Recovery factor as a function of time (Semi-implicit formulation)

Results comparison

A plot of oil pressure as a function of distance along path of flow at 1.5 days for the IMPES and Semi-implicit formulations are shown in Figure 7. The results are equal for practical purposes.

Figure 7. Comparison of pressure solution obtained with the IMPES formulation and Semi-implicit formulation.

The water saturation profiles at 1.5 days obtained with the IMPES and Semi-implicit formulations and with the four cases studied in HW #3 (Buckley-Leverett) are shown in Figure 7. The case that shows a greater numerical dispersion, manifested by a spreader saturation curve is Case 3, the implicit Buckley-Leverett solution with coarse grid. The explicit Buckley-Leverett Case 2 with finer grid represents the case with the lowest numerical dispersion, showing a sharper water front. The solution obtained with the numerical cases based on the solution of the pressure and saturation equation using IMPES and semi-implicit formulations are equal. The numerical solution is close to HW #3 Case 2 solution, the one with the sharper front (lowest numerical dispersion).

Figure 8.

Addendum A.1 - MATLAB script for IMPES formulation

%===== PEGN513_HW9_v1.m

%

% 1D solution of pressure equation implicitly and saturation equation

% explicitly (IMPES formulation)

%

%

% by Luis Zerpa

% Nov 2009

%

clc,clear all,close all

dt = 0.1; % days

tmax = 10; % days

nTimeSteps = tmax/dt+1;

% GRID

IMAX = 10; % ft

dx = 10; % ft

dy = 10; % ft

dz = 10; % ft

VR = dx*dy*dz;

% ROCK PROPERTIES

krw_ast = 0.1; krow_ast = 0.7; nw = 1.5; now = 2.5; Sorw = 0.3; Swr = 0.25;

alpha = 0.8; Swx = 0.5; Pcwo_max = 5; Pcwo_min = -5;

porosity = 0.2;

permeability = 100; % md

% FLUID PROPERTIES

water_visc = 0.6; % cp

oil_visc = 2.4; % cp

gamma_w = 0.433; % psi/ft

gamma_o = 0.433*0.8; % psi/ft

% INJECTION RATE

qt = 200; % ft^3/day

% INITIAL AND BOUNDARY CONDITIONS

Pi = 2000; % psia

Pwell = 1900; % psia

Swi = 0.25;

% COMPRESSIBILITIES

Cphi = 3e-6; % 1/psi

Cw = 4e-6; % 1/psi

Co = 10e-6; % 1/psi

% INITIALIZATION

Po = Pi*ones(1,IMAX);

Sw = Swi*ones(1,IMAX);

So = 1-Sw;

% CAPILLARY PRESSURE

Pcwo = getPcwo(Sw,alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min);

Pw = Po-Pcwo;

% WELL INDEX

[krw(1,IMAX),kro(1,IMAX)] = getRelPerm(Sw(1,IMAX),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(1,IMAX) = krw(1,IMAX)/water_visc;

oil_mob(1,IMAX) = kro(1,IMAX)/oil_visc;

WI = 0.006328*permeability*(water_mob(1,IMAX)+oil_mob(1,IMAX))*dy*dz/(dx/2);

WIw = WI.*water_mob(1,IMAX)./(water_mob(1,IMAX)+oil_mob(1,IMAX));

for n = 1:nTimeSteps

Ct(n,:) = Sw(n,:)*Cw + (1-Sw(n,:))*Co + Cphi;

% SOLVE PRESSURE EQUATION

for i = 1:IMAX

if i == 1 % First node

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

Tt_R = 0.006328*permeability*(water_mob(n,i)+oil_mob(n,i))*dy*dz/dx;

Tw_R = Tt_R*water_mob(n,i)/(water_mob(n,i)+oil_mob(n,i));

D(i,1) = 0;

E(i,1) = -(Tt_R + VR*porosity*Ct(n,i)/dt);

F(i,1) = Tt_R;

R(i,1) = Tw_R*(Pcwo(n,i+1)-Pcwo(n,i))...

- qt - (Tt_R*(Po(n,i+1)-Po(n,i)));

elseif i == IMAX % Last node

Tt_L = Tt_R;

Tw_L = Tw_R;

Tt_R = 0;

Tw_R = 0;

D(i,1) = Tt_L;

E(i,1) = -(Tt_L + VR*porosity*Ct(n,i)/dt + WI(n,1));

F(i,1) = 0;

R(i,1) = - Tw_L*(Pcwo(n,i)-Pcwo(n,i-1)) + WI(n)*(Po(n,i) - Pwell) +(Tt_L*(Po(n,i)-Po(n,i-1)));

else % Intermediate nodes

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

Tt_L = Tt_R;

Tw_L = Tw_R;

Tt_R = 0.006328*permeability*(water_mob(n,i)+oil_mob(n,i))*dy*dz/dx;

Tw_R = Tt_R*water_mob(n,i)/(water_mob(n,i)+oil_mob(n,i));

D(i,1) = Tt_L;

E(i,1) = -(Tt_L + Tt_R + VR*porosity*Ct(n,i)/dt);

F(i,1) = Tt_R;

R(i,1) = Tw_R*(Pcwo(n,i+1)-Pcwo(n,i)) - Tw_L*(Pcwo(n,i)-Pcwo(n,i-1)) -(Tt_R*(Po(n,i+1)-Po(n,i)) - Tt_L*(Po(n,i)-Po(n,i-1)));

end

end

dPo = tdma(D,E,F,R);

Po(n+1,:) = dPo' + Po(n,:);   %======================================================================

% SOLVE SATURATION EQUATION EXPLICITLY

for i = 1: IMAX

if i == 1 % First node

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

Tw_R = 0.006328*permeability*(water_mob(n,i))*dy*dz/dx;

Sw(n+1,i) = (Tw_R*(Po(n+1,i+1)-Po(n+1,i)) - Tw_R*(Pcwo(n,i+1)-Pcwo(n,i)) + qt)*dt/(VR*porosity)...

- Sw(n,i)*(Cphi+Cw)*(Po(n+1,i)-Po(n,i)) + Sw(n,i);

elseif i == IMAX % Last node

Tt_L = Tt_R;

Tw_L = Tw_R;

Tt_R = 0;

Tw_R = 0;

Sw(n+1,i) = (- Tw_L*(Po(n+1,i)-Po(n+1,i-1)) ...

+ Tw_L*(Pcwo(n,i)-Pcwo(n,i-1))...

- WIw(n,1)*(Po(n+1,i)-Pwell))*dt/(VR*porosity)...

- Sw(n,i)*(Cphi+Cw)*(Po(n+1,i)-Po(n,i)) + Sw(n,i);

else % Intermediate nodes

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

Tw_L = Tw_R;

Tw_R = 0.006328*permeability*(water_mob(n,i))*dy*dz/dx;

Sw(n+1,i) = (Tw_R*(Po(n+1,i+1)-Po(n+1,i)) - Tw_L*(Po(n+1,i)-Po(n+1,i-1)) - Tw_R*(Pcwo(n,i+1)-Pcwo(n,i)) + Tw_L*(Pcwo(n,i)-Pcwo(n,i-1)))*dt/(VR*porosity) - Sw(n,i)*(Cphi+Cw)*(Po(n+1,i)-Po(n,i)) + Sw(n,i);

end

end

Pcwo(n+1,:) = getPcwo(Sw(n+1,:),alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min);

Pw(n+1,:) = Po(n+1,:)-Pcwo(n+1,:);

% Well index

[krw(n+1,IMAX),kro(n+1,IMAX)] = getRelPerm(Sw(n+1,IMAX),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n+1,IMAX) = krw(n+1,IMAX)/water_visc;

oil_mob(n+1,IMAX) = kro(n+1,IMAX)/oil_visc;

WI(n+1,1) = 0.006328*permeability*(water_mob(n+1,IMAX)+oil_mob(n+1,IMAX))*dy*dz/(dx/2);

WIw(n+1,1) = WI(n+1,1).*water_mob(n+1,IMAX)./(water_mob(n+1,IMAX)+oil_mob(n+1,IMAX));

end

figure

plot([dx/2:dx:dx*IMAX-dx/2],Sw([1:5:100],:),'linewidth',2)

xlabel('X coordinate (ft)')

ylabel('Water saturation (fraction)')

axis([0,IMAX*dx,0,1])

axis square

qo = WI.*(Po(:,IMAX)-Pwell).*oil_mob(:,IMAX)./(water_mob(:,IMAX)+oil_mob(:,IMAX));

cum_oil = 0;

for i = 2:length(qo)

cum_oil(i,1) = qo(i-1)*dt + cum_oil(i-1);

end

OOIP = sum(VR*porosity*(1-Sw(1,:)));

RecoveryFactor = cum_oil/OOIP*100;

figure

plot([0:dt:tmax],RecoveryFactor(1:101,1),'linewidth',2)

xlabel('Time (days)')

ylabel('Recovery Factor (%)')

Addendum A.2 - MATLAB script for Semi-implicit formulation

%===== PEGN513_HW11_v0.m

%

% 1D solution of the Pressure and Saturation equations using the partially

% implicit formulation

%

% Luis Zerpa

% 15 Nov 2009

%

clc,clear all,close all

dt = 0.1; % days

tmax = 10; % days

nTimeSteps = tmax/dt+1;

IMAX = 10; % ft

dx = 10; % ft

dy = 10; % ft

dz = 10; % ft

VR = dx*dy*dz;

krw_ast = 0.1; krow_ast = 0.7; nw = 1.5; now = 2.5; Sorw = 0.3; Swr = 0.25; alpha = 0.8; Swx = 0.5; Pcwo_max = 5; Pcwo_min = -5;

water_visc = 0.6; % cp

oil_visc = 2.4; % cp

gamma_w = 0.433; % psi/ft

gamma_o = 0.433*0.8; % psi/ft

porosity = 0.2;

qt = 200; % ft^3/day

permeability = 100; % md

Pi = 2000; % psia

Pwell = 1900; % psia

Swi = 0.25;

Cphi = 3e-6; % 1/psi

Cw = 4e-6; % 1/psi

Co = 10e-6; % 1/psi

% initialization

Po = Pi*ones(1,IMAX);

Sw = Swi*ones(1,IMAX);

So = 1-Sw;

Pcwo = getPcwo(Sw,alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min);

Pw = Po-Pcwo;

% Well index

[krw(1,IMAX),kro(1,IMAX)] = getRelPerm(Sw(1,IMAX),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(1,IMAX) = krw(1,IMAX)/water_visc;

oil_mob(1,IMAX) = kro(1,IMAX)/oil_visc;

WI = 0.006328*permeability*(water_mob(1,IMAX)+oil_mob(1,IMAX))*dy*dz/(dx/2);

WIw = WI.*water_mob(1,IMAX)./(water_mob(1,IMAX)+oil_mob(1,IMAX));

for n = 1:nTimeSteps

Ct(n,:) = Sw(n,:)*Cw + (1-Sw(n,:))*Co + Cphi;

% SOLVE PRESSURE EQUATION

for i = 1:IMAX

if i == 1 % First node

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

Tt_R = 0.006328*permeability*(water_mob(n,i)+oil_mob(n,i))*dy*dz/dx;

Tw_R = Tt_R*water_mob(n,i)/(water_mob(n,i)+oil_mob(n,i));

D(i,1) = 0;

E(i,1) = -(Tt_R + VR*porosity*Ct(n,i)/dt);

F(i,1) = Tt_R;

R(i,1) = Tw_R*(Pcwo(n,i+1)-Pcwo(n,i))...

- qt - (Tt_R*(Po(n,i+1)-Po(n,i)));

elseif i == IMAX % Last node

Tt_L = Tt_R;

Tw_L = Tw_R;

Tt_R = 0;

Tw_R = 0;

D(i,1) = Tt_L;

E(i,1) = -(Tt_L + VR*porosity*Ct(n,i)/dt + WI(n,1));

F(i,1) = 0;

R(i,1) = - Tw_L*(Pcwo(n,i)-Pcwo(n,i-1)) + WI(n)*(Po(n,i) - Pwell+(Tt_L*(Po(n,i)-Po(n,i-1)));

else % Intermediate nodes

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

Tt_L = Tt_R;

Tw_L = Tw_R;

Tt_R = 0.006328*permeability*(water_mob(n,i)+oil_mob(n,i))*dy*dz/dx;

Tw_R = Tt_R*water_mob(n,i)/(water_mob(n,i)+oil_mob(n,i));

D(i,1) = Tt_L;

E(i,1) = -(Tt_L + Tt_R + VR*porosity*Ct(n,i)/dt);

F(i,1) = Tt_R;

R(i,1) = Tw_R*(Pcwo(n,i+1)-Pcwo(n,i)) - Tw_L*(Pcwo(n,i)-Pcwo(n,i-1)) -(Tt_R*(Po(n,i+1)-Po(n,i)) - Tt_L*(Po(n,i)-Po(n,i-1)));

end

end

% Solve Pressure equations linear system using the Thomas Algorithm for

% tri-diagonal matrices

dtPo = tdma(D,E,F,R);

Po(n+1,:) = dtPo' + Po(n,:);

%=======================================================================

% SOLVE SATURATION EQUATION IMPLICITLY

for i = 1: IMAX

if i == 1 % First node

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

Tw_R = 0.006328*permeability*(water_mob(n,i))*dy*dz/dx;

dPcwo_dSw_R = getdPcwo_dSw(Sw(n,i),alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min);

Ds(i,1) = 0;

Es(i,1) = Tw_R*dPcwo_dSw_R - VR*porosity/dt;

Fs(i,1) = -Tw_R*dPcwo_dSw_R;

Rs(i,1) = VR*porosity*(Sw(n,i)*(Cphi + Cw)*(Po(n+1,i)-Po(n,i)))/dt - Tw_R*(Po(n+1,i+1)-Po(n+1,i)) - qt...

+ Tw_R*dPcwo_dSw_R*(Sw(n,i+1)-Sw(n,i));

elseif i == IMAX % Last node

Tt_L = Tt_R;

Tw_L = Tw_R;

Tt_R = 0;

Tw_R = 0;

dPcwo_dSw_L = dPcwo_dSw_R;

Ds(i,1) = -Tw_L*dPcwo_dSw_L;

Es(i,1) = Tw_L*dPcwo_dSw_L - VR*porosity/dt;

Fs(i,1) = 0;

Rs(i,1) = VR*porosity*(Sw(n,i)*(Cphi + Cw)*(Po(n+1,i)-Po(n,i)))/dt + Tw_L*(Po(n+1,i)-Po(n+1,i-1)) + WIw(n,1)*(Po(n+1,i)-Pwell) - Tw_L*dPcwo_dSw_L*(Sw(n,i)-Sw(n,i-1));

else % Intermediate nodes

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

dPcwo_dSw_L = dPcwo_dSw_R;

dPcwo_dSw_R = getdPcwo_dSw(Sw(n,i),alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min);

Tw_L = Tw_R;

Tw_R = 0.006328*permeability*(water_mob(n,i))*dy*dz/dx;

Ds(i,1) = -Tw_L*dPcwo_dSw_L;

Es(i,1) = Tw_L*dPcwo_dSw_L + Tw_R*dPcwo_dSw_R - VR*porosity/dt;

Fs(i,1) = -Tw_R*dPcwo_dSw_R;

Rs(i,1) = VR*porosity*(Sw(n,i)*(Cphi + Cw)*(Po(n+1,i)-Po(n,i)))/dt - Tw_R*(Po(n+1,i+1)-Po(n+1,i)) + Tw_L*(Po(n+1,i)-Po(n+1,i-1)) + Tw_R*dPcwo_dSw_R*(Sw(n,i+1)-Sw(n,i)) - Tw_L*dPcwo_dSw_L*(Sw(n,i)-Sw(n,i-1));

end

end

% Solve Saturation equations linear system using the Thomas Algorithm for tri-diagonal matrices

dtSw = tdma(Ds,Es,Fs,Rs);

Sw(n+1,:) = dtSw' + Sw(n,:);

Pcwo(n+1,:) = getPcwo(Sw(n+1,:),alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min);

Pw(n+1,:) = Po(n+1,:)-Pcwo(n+1,:);

% Well index

[krw(n+1,IMAX),kro(n+1,IMAX)] = getRelPerm(Sw(n+1,IMAX),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n+1,IMAX) = krw(n+1,IMAX)/water_visc;

oil_mob(n+1,IMAX) = kro(n+1,IMAX)/oil_visc;

WI(n+1,1) = 0.006328*permeability*(water_mob(n+1,IMAX)+oil_mob(n+1,IMAX))*dy*dz/(dx/2);

WIw(n+1,1) = WI(n+1,1).*water_mob(n+1,IMAX)./(water_mob(n+1,IMAX)+oil_mob(n+1,IMAX));

end

figure

plot([dx/2:dx:dx*IMAX-dx/2],Po([1:5:100],:),'linewidth',2)

xlabel('X coordinate (ft)')

ylabel('Oil Pressure (psi)')

figure

plot([dx/2:dx:dx*IMAX-dx/2],Sw([1:5:100],:),'linewidth',2)

xlabel('X coordinate (ft)')

ylabel('Water saturation (fraction)')

axis([0,IMAX*dx,0,1])

axis square

qo = WI.*(Po(:,IMAX)-Pwell).*oil_mob(:,IMAX)./(water_mob(:,IMAX)+oil_mob(:,IMAX));

cum_oil = 0;

for i = 2:length(qo)

cum_oil(i,1) = qo(i-1)*dt + cum_oil(i-1);

end

OOIP = sum(VR*porosity*(1-Sw(1,:)));

RecoveryFactor = cum_oil/OOIP*100;

figure

plot([0:dt:tmax],RecoveryFactor(1:101,1),'linewidth',2)

xlabel('Time (days)')

ylabel('Recovery Factor (%)')

Sub-routines

%===== getRelPerm.m

%

% by Luis Zerpa

% Nov 2009

%

function [krw,kro] = getRelPerm(Sw,Sorw,Swr,nw,now,krw_ast,krow_ast)

for i = 1:length(Sw)

if Sw(i) < Swr

krw(i) = 0;

else

krw(i) = krw_ast*((Sw-Swr)/(1-Swr-Sorw)).^nw;

end

if 1-Sw(i) < Sorw

kro(i) = 0;

else

kro(i) = krow_ast*((1-Sw-Sorw)/(1-Swr-Sorw)).^now;

end

end

%==== getPcwo.m

%

% by Luis Zerpa

% Nov 2009

%

function [Pcwo] = getPcwo(Sw,alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min)

for i = 1:length(Sw)

if Sw(i) <= Swr

Pcwo(i) = Pcwo_max;

elseif Sw(i) >= 1-Sorw

Pcwo(i) = Pcwo_min;

elseif Sw(i) < Swx & Sw(i) > Swr

Pcwo2 = -alpha*log((Sw(i)-Swr)/(Swx-Swr));

Pcwo(i) = min(Pcwo2,Pcwo_max);

else

Pcwo1 = alpha*log((1-Sw(i)-Sorw)/(1-Swx-Sorw));

Pcwo(i) = max(Pcwo1,Pcwo_min);

end

end

%===== tdma.m

%

% Thomas Algorithm for linear systems with tri-diagonal matrix

%

%  Luis Zerpa

%  January 2004

%

function [x] = tdma(e,f,g,B)

n = length(B);

% Descomposicion

for k = 2:n

e(k) = e(k)/f(k-1);

f(k) = f(k) - e(k)*g(k-1);

end

% Sustitucion hacia adelante

for k = 2:n

B(k) = B(k) - e(k)*B(k-1);

end

% Sustitucion hacia atras

x(n,1) = B(n)/f(n);

for k = n-1:-1:1

x(k) = (B(k) - g(k)*x(k+1))/f(k);

end

U(1,1) = f(1); U(1,2) = g(1); L(1,1) = 1;

L(n,n-1) = e(n); U(n,n) = f(n); L(n,n) = 1;

for i = 1:n-1

L(i,i) = 1;

L(i+1,i) = e(i+1);

U(i,i) = f(i);

U(i,i+1) = g(i);

end

%==== getdPcwo_dSw.m

%

% Derivative of Water-Oil Capillary pressure with respect to water saturation

%

% Luis Zerpa

% 15 Nov 2009

%

function [dPcwo_dSw] = getdPcwo_dSw(Sw,alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min)

for i = 1:length(Sw)

if Sw(i) <= Swr

dPcwo_dSw(i) = 0;

elseif Sw(i) >= 1-Sorw

dPcwo_dSw(i) = 0;

elseif Sw(i) < Swx & Sw(i) > Swr

dPcwo_dSw(i) = -alpha/(Sw(i)-Swr);

else

dPcwo_dSw(i) = -alpha/(1-Sw(i)-Sorw);

end

end