PEGN 513

Reservoir Simulation I

Fall 2009

Homework # 2

Due Date: 9/15/2009

Luis E. Zerpa

Homework #2

The 1D Buckley-Leverett problem was solved using the Explicit Upstream Weighted finite difference formulation. Considering the simplest form for waterflood of a linear core initially saturated with oil and at residual water saturation, the material balance equation (Equation 1) presented by Buckley and Leverett (1941) is,

∂Sw∂t=−ut∂fw∂x	(1)

where,

Sw: water saturation

t: time

x: distance along path of flow

ut: total interstitial velocity, ut=qwAφ

fw: water fractional flow, fw=λwSwλwSw+λoSw

qw: water injection rate

A: cross-sectional area of the core

: porosity

mobility of water is, λwSw=krwSwμw

mobility of oil is, λwSw=krwSwμw

and the relative permeability is calculated as a function of saturation using,

krw=krw∗Sw−Swr1−Sorw−Swrnw

kro=kro∗So−Sorw1−Sorw−Swrno

Considering the space discretization shown in Figure 1, Equation 1 can be expressed in its discretized form (that applies for each space partition) as,

Sw,in+1−Sw,inΔt=−utfw,in−fw,i−1nΔx	(2)

To improve the computation process and avoid using an imaginary grid block at the left of the first node, the following equation is used for the first node,

Sw,1n+1−Sw,1nΔt=−utfw,1n−fw,12nΔx2	(3)

These equations (Eqs. 2 and 3) can be solved explicitly for water saturation at the new time step (n+1).

1

2

3

i-1

i

i+1

IMAX-1

IMAX

i-1/2

i+1/2

IMAX-1/2

1 1/2

1/2

xi

Figure 1. 1-dimensional core space discretization

The Explicit Upstream Weighted Formulation described to solve the 1-dimensional Buckley-Leverett problem was coded in a MATLAB script, shown at the end of this document.

Table 1 shows the values of water saturation for the 10 grid blocks at 10 days.

Table 1. Water saturation values for the 10 grid blocks at 10 days

| Grid block index | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sw (fraction) | 0.68313 | 0.66975 | 0.66058 | 0.65302 | 0.64641 | 0.64045 | 0.63497 | 0.62986 | 0.62503 | 0.62045 |

A plot of water saturation profile as a function of distance along path of flow at every five time steps is show in Figure 2. The initial condition (at t = 0) corresponds to the horizontal line of constant water saturation equal 0.25 (i.e. Swr). The figure shows an increase of water saturation with time and how the water injection front propagates along the 1-dimensional core.

Figure 2. Water saturation profile along the path of flow at every five time steps

MATLAB script

%===== PEGN513_HW2.m

%

% 1D Buckley-Leverett solution using Emplicit Upstream Wighted Formulation

%

% by Luis Zerpa

% September 10th 2009

%

clc,close all,clear all

% INPUT

krw_ast = 0.10; krow_ast = 0.70; nw = 1.5; no = 2.5; Sorw = 0.30;

Swr = 0.25;

viscosity_water = 0.6; % cp

viscosity_oil = 2.4; % cp

% 1D core

area = 100; % ft^2

porosity = 0.2;

inj_rate = 200; % ft^3/day

IMAX = 10;

delta_x = 10; % ft

timeStep = 0.1; % days

MAXtimeSteps = 100;

% Initialization

Sw = Swr*ones(1,IMAX);

So = (1-Swr)*ones(1,IMAX);

% Calculations

% Total interstitial velocity

ut = inj_rate/(area*porosity);

% 1D Buckley-Leverett solution using Emplicit Upstream Wighted Formulation

for n = 1:MAXtimeSteps

Fw(n,:) = FractionalFlow1D(Sw(n,:),Swr,Sorw,krw_ast,krow_ast,nw,no,...

viscosity_water,viscosity_oil);

Fw_half = 1;

Sw(n+1,1) = Sw(n,1) - timeStep*ut*(Fw(n,1) - Fw_half)/(delta_x/2);

Sw(n+1,2:IMAX) = Sw(n,2:IMAX) - timeStep*ut*(Fw(n,2:IMAX) - Fw(n,1:IMAX-1))/(delta_x);

end

% Plot Water Saturation as a function of time

figure, hold on

plot([delta_x/2:delta_x:delta_x*(IMAX-1/2)],Sw([1:5:MAXtimeSteps],:),'p-')

axis([0,IMAX*delta_x,0,1])

axis square

xlabel('Distance along path of flow (ft)')

ylabel('Water Saturation (fraction)')

grid on

MATLAB function to calculate fractional flow

%===== FractionalFlow1D.m

%

% Calculates fractional flow in 1D

%

% by Luis Zerpa

% September 10th 2009

%

function [Fw] = FractionalFlow1D(Sw,Swr,Sorw,krw_ast,krow_ast,nw,no,...

viscosity_water,viscosity_oil)

% Relative permeabilities

krw = krw_ast*((Sw - Swr)/(1-Sorw-Swr)).^nw;

krow = krow_ast*((1 - Sw - Sorw)/(1-Sorw-Swr)).^no;

% Mobilities

mobility_water = krw/viscosity_water;

mobility_oil = krow/viscosity_oil;

% Water Fractional flow

Fw = mobility_water./(mobility_water + mobility_oil);