PEGN 513

Reservoir Simulation I

Fall 2009

Homework # 5

Due Date: 10/2/2009

Luis E. Zerpa

Homework #5

a) Plot of Capillary Pressure curve

Figure 1 shows a plot of capillary pressure as a function of water saturation for the following fluid system:

krw* = 0.1

krow* = 0.7

nw = 1.5

no = 0.25

Sorw = 0.30

Swr = 0.25

 = 0.8

Swx = 0.5

Pcwo,max = 5 psi

Pcwo,min = - 5 psi

w = 0.433 psi/ft

o = 0.433*0.8 psi/ft

w = 0.6 cp

o = 2.4 cp

k = 20 md

 = 0.20

uT = 10 ft/day

Figure 1. Capillary pressure as a function of water saturation

For the calculation of the capillary pressure the following equations were used,

where,

The calculation procedure was coded in a Matlab script shown at the end of this report.

b) Calculation of the water saturation at the injection front

The water saturation at the injection front, Swf, is defined as the point of tangency on the fractional flow curve, fw, of a straight line drawn from the residual water saturation, Swr. To determine Swf, a procedure that minimizes the difference between the slope of a line from Swr and the derivative of the fractional flow function was written in Matlab. This optimization problem can be stated as,

Find Sw that minimize fwSwSw−Swr−∂fw∂Sw2, subject to Swr < Sw < 1 - Sorw

Obtaining the water saturation at the injection front, Swf = 0.5598, with a corresponding fractional flow of fwf = 0.8576. Figure 2 shows the plot of water fractional flow as a function of water saturation, indicating the point of tangency with the line drawn from Swr.

Figure 2. Water fractional flow as a function of water saturation, indicating point of tangency at Swf

c) Calculation of the terms in the 1-D Buckley-Leverett equation evaluated at the Swf

The capillary term,

−DpcuTφ∂Sw∂x

was calculated considering the definition given for Dpc,

Dpc=−0.006328λok∂Pcwo∂Sw

where,

∂Pcwo∂Sw=∂∂Swαln1−Sw−Sorw1−Swx−Sorw=α1−Swx−Sorw1−Sw−Sorw−11−Swx−Sorw

∂Pcwo∂Sw=0.81−0.5−0.301−0.5598−0.30−11−0.5−0.30=−5.706

and,

Dpc=−0.006328⋅0.0158⋅20⋅−5.706=0.01141

∂Sw∂x=−Swf−SwrLmix

Lmix=3.6230.48Dpctmix=3.6230.480.01141⋅1.5=0.01554

∂Sw∂x=−0.5598−0.250.01554=−19.936

Then the capillary term is,

−DpcuTφ∂Sw∂x=−0.0114110⋅0.20−19.936=0.1138

The gravity term,

0.006328λoγw−γokuTφ∂D∂x

∂D∂x=−sinθ=−sin3=−0.05234

Then the gravity term is,

0.006328λoγw−γokuTφ∂D∂x=0.0063280.01580.433−0.34642010⋅0.20⋅−0.05234=−4.5311×10−6

It can be concluded that the viscous term (= 1) is the more important for the case studied at the injection front saturation, while the gravity term (-4.5388x10-6) is represented by a lower value (almost insignificant because the lower dip angle), and the capillary term (0.1138) having some influence on the response of the 1-D Buckley-Leverett problem.

MATLAB script used for the calculations

%===== PEGN513_HW5.m

%

% a) Create a plot of the capillary pressure curve

% b) Calculate Sw_front

% c) Calculate the value of each of the terms in the 1-D

% Buckley-Leverett equation evaluated at Sw_front

%

% by Luis Zerpa

% October 1th 2009

%

clc,close all,clear all

% INPUT

krw_ast = 0.10; krow_ast = 0.70; nw = 1.5; no = 2.5; Sorw = 0.30;

Swr = 0.25; alpha = 0.8; Swx = 0.5;

Pcwo_max = 5; % psi

Pcwo_min = -5; % psi

viscosity_water = 0.6; % cp

viscosity_oil = 2.4; % cp

gamma_w = 0.433; % psi/ft

gamma_o = 0.433*0.8; % psi/ft

permeability = 20; % md

porosity = 0.2; % fraction

ut = 10; % Total interstitial velocity, ft/day

t_mix = 1.5; % day

% a) Plot of the capillary pressure curve

Sw = [0:0.001:1];

for i = 1:length(Sw)

if Sw(i) <= Swr

Pcwo(i) = Pcwo_max;

elseif Sw(i) >= (1-Sorw)

Pcwo(i) = Pcwo_min;

elseif Sw(i) >= Swr & Sw(i) <= Swx

Pcwo_2 = -alpha*(Swx-Swr)/(1-Swx-Sorw)*log((Sw(i)-Swr)/(1-Swx-Swr));

Pcwo(i) = min(Pcwo_2,Pcwo_max);

else

Pcwo_1 = alpha*log((1-Sw(i)-Sorw)/(1-Swx-Sorw));

Pcwo(i) = max(Pcwo_1,Pcwo_min);

end

end

figure

plot(Sw,Pcwo)

xlabel('Water Saturation, fraction')

ylabel('Oil-Water Capillary Pressure, psi')

grid on

axis square

% b) Calculate Sw_front

Fw = FractionalFlow1D([Swr:0.001:1-Sorw],Swr,Sorw,krw_ast,krow_ast,nw,no,...

viscosity_water,viscosity_oil);

figure

plot([Swr:0.001:1-Sorw],Fw,'LineWidth',2)

xlabel('Water Saturation, fraction')

ylabel('Water Fractional Flow, fraction')

grid on

axis([0,1,0,1])

axis square

hold on

% Look for the intersection of the straight line with Fw curve, solving an

% optimization problem using the MATLAB fmincon function that implements

% the optimization algorithm called Sequential Quadratic Programming

[Sw_front,Fval] = fmincon('Residual',0.5,[],[],[],[],Swr,1-Sorw);

% Calculates de fractional flow at the front saturation

Fw_front = FractionalFlow1D(Sw_front,Swr,Sorw,krw_ast,krow_ast,nw,no,...

viscosity_water,viscosity_oil);

% Plot tangency plot and the breakthrough tangent line on the fractional

% flow curve

plot(Sw_front,Fw_front,'r.','MarkerSize',20)

plot([Swr,Sw_front,(1+Fw_front*Swr/(Sw_front-Swr))*(Sw_front-Swr)/Fw_front],[0,Fw_front,1],'r--','LineWidth',2)

% c) Calculate the value of each of the terms in the 1-D

% Buckley-Leverett equation evaluated at Sw_front

% Derivative of Pcwo with respect to Sw at Sw_front

dPcwo_dSw = alpha*(1-Swx-Sorw)/(1-Sw_front-Sorw)*(-1/(1-Swx-Sorw));

% Dpc

krow = krow_ast*((1 - Sw_front - Sorw)/(1-Sorw-Swr)).^no;

mobility_oil = krow/viscosity_oil;

Dpc = -0.006328*mobility_oil*permeability*dPcwo_dSw;

Lmix = 3.62/30.48*sqrt(Dpc*t_mix);

dSw_dx = -(Sw_front - Swr)/Lmix;

% Capillary term

Capillary_term = -Dpc*dSw_dx/(ut*porosity);

% Gravity term

dD_dx = -sin(3*pi/180);

Gravity_term = 0.006328*mobility_oil*(gamma_w-gamma_o)*permeability/(ut*porosity)*dD_dx;

Sub-routines

%===== Residual.m

%

% Difference between the fractional flow function and the straight line

% that intersects Fw curve at frontal velocity point.

% To determine the intersection between these two functions using an

% optimization algorithm

function [R] = Residual(Sw)

krw_ast = 0.10; krow_ast = 0.70; nw = 1.5; no = 2.5; Sorw = 0.30;

Swr = 0.25; alpha = 0.8; Swx = 0.5;

viscosity_water = 0.6; % cp

viscosity_oil = 2.4; % cp

Fw = FractionalFlow1D(Sw,Swr,Sorw,krw_ast,krow_ast,nw,no,...

viscosity_water,viscosity_oil);

dFw_dSw = DerivativeFractionalFlow_Sw(Sw,Swr,Sorw,krw_ast,krow_ast,nw,no,...

viscosity_water,viscosity_oil);

R = (Fw/(Sw-Swr) - dFw_dSw)^2;

%===== FractionalFlow1D.m

% Calculates fractional flow in 1D

%

% by Luis Zerpa

% September 10th 2009

%

function [Fw] = FractionalFlow1D(Sw,Swr,Sorw,krw_ast,krow_ast,nw,no,...

viscosity_water,viscosity_oil)

% Relative permeabilities

krw = krw_ast*((Sw - Swr)/(1-Sorw-Swr)).^nw;

krow = krow_ast*((1 - Sw - Sorw)/(1-Sorw-Swr)).^no;

% Mobilities

mobility_water = krw/viscosity_water;

mobility_oil = krow/viscosity_oil;

% Water Fractional flow

Fw = mobility_water./(mobility_water + mobility_oil);