PEGN 513

Reservoir Simulation I

Fall 2009

Homework # 8

Due Date: 11/03/2009

Luis E. Zerpa

Homework #8

a) Finite difference expansion of the 1D pressure equation.

1D pressure equation,

0.006328∂∂xkλT∂Po∂x−λwγw+λoγo∂D∂x−λw∂Pcwo∂x+qT=φCT∂Po∂t

Expanding the terms of the left hand side individually,

0.006328∂∂xkλT∂Po∂x0.006328ΔxikλTni+12Po,i+1n+1−Po,in+1Δxi+12−kλTni−12Po,in+1−Po,i−1n+1Δxi−12

multiplying by the volume of the grid block, VRi = xyz

0.006328kλTnΔxi+12ΔyΔziPo,i+1n+1−Po,in+1−0.006328kλTnΔxi−12ΔyΔziPo,in+1−Po,i−1n+1

Using the definition of total transmissibility,

TTx,i+12n=0.006328kλTnΔxi+12ΔyΔzi

TTx,i+12nPo,i+1n+1−Po,in+1−TTx,i−12nPo,in+1−Po,i−1n+1

For the gravity term,

−0.006328∂∂xkλwγw+λoγo∂D∂x−0.006328Δxikλwnγw+λonγoΔxi+12Di+1−Di−kλwnγw+λonγoΔxi−12Di−Di−1

multiplying by the volume of the grid block, VRi = xyz

−0.006328ΔyΔzikλwnγw+λonγoΔxi+12Di+1−Di−kλwnγw+λonγoΔxi−12Di−Di−1

Using the definition of water and oil transmissibility,

Twx,i+12n=0.006328kλwnΔxi+12ΔyΔzi,         Tox,i+12n=0.006328kλonΔxi+12ΔyΔzi

−Twxnγw+Toxnγoi+12Di+1−Di−Twxnγw+Toxnγoi−12Di−Di−1

For the capillary term,

−0.006328∂∂xkλw∂Pcwo∂x

−0.006328ΔxikλwnΔxi+12Pcwo,i+1n−Pcwo,in−kλwnΔxi−12Pcwo,in−Pcwo,i−1n

multiplying by the volume of the grid block, VRi = xyz

−0.006328ΔyΔzikλwnΔxi+12Pcwo,i+1n−Pcwo,in−kλwnΔxi−12Pcwo,in−Pcwo,i−1n

Using the definition of water transmissibility,

−Twx,i+12nPcwo,i+1n−Pcwo,in−Twx,i−12nPcwo,in−Pcwo,i−1n

Expanding the right hand side,

φCT∂Po∂t

φiCTΔtPo,in+1−Po,in

multiplying by the volume of the grid block, VRi = xyz

VRiφiCTΔtPo,in+1−Po,in

The expanded 1D pressure equation is,

TTx,i+12nPo,i+1n+1−Po,in+1−TTx,i−12nPo,in+1−Po,i−1n+1−Twxnγw+Toxnγoi+12Di+1−Di−Twxnγw+Toxnγoi−12Di−Di−1−Twx,i+12nPcwo,i+1n−Pcwo,in−Twx,i−12nPcwo,in−Pcwo,i−1n+qT,i=VRiφiCTΔtPo,in+1−Po,in

Collecting terms for all P at n+1, and moving other terms to the right hand side,

TTx,i−12nPo,i−1n+1−TTx,i−12n+TTx,i+12n+VRiφiCTΔtPo,in+1+TTx,i+12nPo,i+1n+1=−VRiφiCTΔtPo,in+Twxnγw+Toxnγoi+12Di+1−Di−Twxnγw+Toxnγoi−12Di−Di−1+Twx,i+12nPcwo,i+1n−Pcwo,in−Twx,i−12nPcwo,in−Pcwo,i−1n−qT,i

b) 5-node problem

For i = 1

−TTx,112n+VR1φ1CTΔtPo,1n+1+TTx,112nPo,2n+1=−VR1φ1CTΔtPo,1n+Twxnγw+Toxnγo112D2−D1+Twx,112nPcwo,2n−Pcwo,2n−qT,1

For i = 2

TTx,112nPo,1n+1−TTx,112n+TTx,212n+VR2φ2CTΔtPo,2n+1+TTx,212nPo,3n+1=−VR2φ2CTΔtPo,2n+Twxnγw+Toxnγo212D3−D2−Twxnγw+Toxnγo112D2−D1+Twx,212nPcwo,3n−Pcwo,2n−Twx,112nPcwo,2n−Pcwo,1n−qT,2

For i = 3

TTx,212nPo,2n+1−TTx,212n+TTx,312n+VR3φ3CTΔtPo,3n+1+TTx,312nPo,4n+1=−VR3φ3CTΔtPo,3n+Twxnγw+Toxnγo312D4−D3−Twxnγw+Toxnγo212D3−D2+Twx,312nPcwo,4n−Pcwo,3n−Twx,212nPcwo,3n−Pcwo,2n−qT,3

For i = 4

TTx,312nPo,3n+1−TTx,312n+TTx,412n+VR4φ4CTΔtPo,4n+1+TTx,412nPo,5n+1=−VR4φ4CTΔtPo,4n+Twxnγw+Toxnγo412D5−D4−Twxnγw+Toxnγo312D4−D3+Twx,412nPcwo,5n−Pcwo,4n−Twx,312nPcwo,4n−Pcwo,3n−qT,4

For i = 5

TTx,412nPo,4n+1−TTx,412n+VR5φ5CTΔtPo,5n+1=−VR5φ5CTΔtPo,5n+−Twxnγw+Toxnγo412D5−D4+−Twx,412nPcwo,5n−Pcwo,4n−qT,5