PEGN 513

Reservoir Simulation I

Fall 2009

Homework # 9

Due Date: 11/10/2009

Luis E. Zerpa

Homework #9

a) Summary of finite different equations.

1D pressure equation,

0.006328∂∂xkλT∂Po∂x−λwγw+λoγo∂D∂x−λw∂Pcwo∂x+qT=φCT∂Po∂t

The expanded 1D pressure equation is,

TTx,i−12nPo,i−1n+1−TTx,i−12n+TTx,i+12n+VRiφiCTΔtPo,in+1+TTx,i+12nPo,i+1n+1=−VRiφiCTΔtPo,in+Twxnγw+Toxnγoi+12Di+1−Di−Twxnγw+Toxnγoi−12Di−Di−1+Twx,i+12nPcwo,i+1n−Pcwo,in−Twx,i−12nPcwo,in−Pcwo,i−1n−qT,i

The production rate is calculated with the following expression as a function of well index,

qT,i=−WIinPo,in+1−Pwelln

Where the well index is given by,

WIin=0.006328kiλTinΔyiΔziΔxi2

Substituting into the expanded 1D pressure equation for the nodes with a producer,

TTx,i−12nPo,i−1n+1−TTx,i−12n+TTx,i+12n+VRiφiCTΔt+WIinPo,in+1+TTx,i+12nPo,i+1n+1=−VRiφiCTΔtPo,in+Twxnγw+Toxnγoi+12Di+1−Di−Twxnγw+Toxnγoi−12Di−Di−1+Twx,i+12nPcwo,i+1n−Pcwo,in−Twx,i−12nPcwo,in−Pcwo,i−1n−WIinPwelln

Total transmissibility,

TTx,i+12n=0.006328kλTnΔxi+12ΔyΔzi

Water transmissibility,

Twx,i+12n=0.006328kλwnΔxi+12ΔyΔzi

Oil transmissibility,

Tox,i+12n=0.006328kλonΔxi+12ΔyΔzi

1D saturation equation,

∂∂xkλw∂Po∂x−γw∂D∂x−∂Pcwo∂x+qw=φSwCφ+Cw∂Po∂t+∂Sw∂t

The expanded 1D saturation equation is,

Twx,i+12nPo,i+1n+1−Po,in+1−TTx,i−12nPo,in+1−Po,i−1n+1−Twx,i+12nγwDi+1−Di−TTx,i−12nγwDi−Di−1−Twx,i+12nPcwo,i+1n−Pcwo,in−Twx,i−12nPcwo,in−Pcwo,i−1n+qw,iVRφSw,inCφ+CwΔtPo,in+1−Po,in+Sw,in+1−Sw,inΔt

The pressure equation can be solve implicitly, and then the saturation equation can be solve explicitly using,

Sw,in+1=Twx,i+12nPo,i+1n+1−Po,in+1−TTx,i−12nPo,in+1−Po,i−1n+1−Twx,i+12nγwDi+1−Di+TTx,i−12nγwDi−Di−1−Twx,i+12nPcwo,i+1n−Pcwo,in+Twx,i−12nPcwo,in−Pcwo,i−1n+qw,iΔtVRφ+Sw,in1−Cφ+CwPo,in+1−Po,in

In this work the pressure equation was solve using the Thomas algorithm for tri-diagonal matrices, based on the LU decomposition of the matrix and taking advantaged of the matrix structure. A Matlab script was written to implement the solution of the pressure and saturation equation for a 1-dimensional porous medium with 10 blocks in the x direction, and is presented at the end of this report.

b) Plot of Sw vs. x for various times

Figure 1, shows a plot of the water saturation profile as a function of distance along path of flow at every five time steps.

Figure 1. Water saturation profile as a function of x coordinate, for every five time steps

Figure 2 shows a comparison between the solution shown in Figure 1 and the solution of HW #3 Case 3. In this figure the solution every eight time steps is shown, it can be observe that the solution of HW #3 corresponds to a more spread saturation profiles, because of the numerical dispersion of the fully implicit formulation, while the new solution shows a shaper front.

Figure 2. Water saturation profile as a function of x coordinate, for every eight time steps, showing the solution obtained in this homework and the solution of HW #3 Case 3

c) Plot of Recovery Factor vs. time

Figure 3, shows a plot of the recovery factor as a function of time. The total recovery factor at 10 days is 51.45 % OOIP.

Figure 3. Recovery factor as a function of time

MATLAB script used for the calculations

%===== PEGN513_HW9_v0.m

%

% 1D solution of pressure equation implicitly and saturation equation

% explicitly

%

%

% by Luis Zerpa

% Nov 2009

%

clc,clear all,close all

dt = 0.1; % days

tmax = 10; % days

nTimeSteps = tmax/dt+1;

% GRID

IMAX = 10; % ft

dx = 10; % ft

dy = 10; % ft

dz = 10; % ft

VR = dx*dy*dz;

% ROCK PROPERTIES

krw_ast = 0.1; krow_ast = 0.7; nw = 1.5; now = 2.5; Sorw = 0.3; Swr = 0.25;

alpha = 0.8; Swx = 0.5; Pcwo_max = 5; Pcwo_min = -5;

porosity = 0.2;

permeability = 100; % md

% FLUID PROPERTIES

water_visc = 0.6; % cp

oil_visc = 2.4; % cp

gamma_w = 0.433; % psi/ft

gamma_o = 0.433*0.8; % psi/ft

% INJECTION RATE

qt = 200; % ft^3/day

% INITIAL AND BOUNDARY CONDITIONS

Pi = 2000; % psia

Pwell = 1900; % psia

Swi = 0.25;

% COMPRESSIBILITIES

Cphi = 3e-6; % 1/psi

Cw = 4e-6; % 1/psi

Co = 10e-6; % 1/psi

% INITIALIZATION

Po = Pi*ones(1,IMAX);

Sw = Swi*ones(1,IMAX);

So = 1-Sw;

% CAPILLARY PRESSURE

Pcwo = getPcwo(Sw,alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min);

Pw = Po-Pcwo;

% WELL INDEX

[krw(1,IMAX),kro(1,IMAX)] = getRelPerm(Sw(1,IMAX),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(1,IMAX) = krw(1,IMAX)/water_visc;

oil_mob(1,IMAX) = kro(1,IMAX)/oil_visc;

WI = 0.006328*permeability*(water_mob(1,IMAX)+oil_mob(1,IMAX))*dy*dz/(dx/2);

WIw = WI.*water_mob(1,IMAX)./(water_mob(1,IMAX)+oil_mob(1,IMAX));

for n = 1:nTimeSteps

Ct(n,:) = Sw(n,:)*Cw + (1-Sw(n,:))*Co + Cphi;

% SOLVE PRESSURE EQUATION

for i = 1:IMAX

if i == 1 % First node

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

Tt_R = 0.006328*permeability*(water_mob(n,i)+oil_mob(n,i))*dy*dz/dx;

Tw_R = Tt_R*water_mob(n,i)/(water_mob(n,i)+oil_mob(n,i));

D(i,1) = 0;

E(i,1) = -(Tt_R + VR*porosity*Ct(n,i)/dt);

F(i,1) = Tt_R;

R(i,1) = -VR*porosity*Ct(n,i)*Po(n,i)/dt ...

+ Tw_R*(Pcwo(n,i+1)-Pcwo(n,i))...

- qt;

elseif i == IMAX % Last node

Tt_L = Tt_R;

Tw_L = Tw_R;

Tt_R = 0;

Tw_R = 0;

D(i,1) = Tt_L;

E(i,1) = -(Tt_L + VR*porosity*Ct(n,i)/dt + WI(n,1));

F(i,1) = 0;

R(i,1) = -VR*porosity*Ct(n,i)*Po(n,i)/dt ...

- Tw_L*(Pcwo(n,i)-Pcwo(n,i-1))...

- WI(n)*Pwell;

else % Intermediate nodes

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

Tt_L = Tt_R;

Tw_L = Tw_R;

Tt_R = 0.006328*permeability*(water_mob(n,i)+oil_mob(n,i))*dy*dz/dx;

Tw_R = Tt_R*water_mob(n,i)/(water_mob(n,i)+oil_mob(n,i));

D(i,1) = Tt_L;

E(i,1) = -(Tt_L + Tt_R + VR*porosity*Ct(n,i)/dt);

F(i,1) = Tt_R;

R(i,1) = -VR*porosity*Ct(n,i)*Po(n,i)/dt ...

+ Tw_R*(Pcwo(n,i+1)-Pcwo(n,i)) - Tw_L*(Pcwo(n,i)-Pcwo(n,i-1));

end

end

Po(n+1,:) = tdma(D,E,F,R);

%=======================================================================

% SOLVE SATURATION EQUATION EXPLICITLY

for i = 1: IMAX

if i == 1 % First node

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

Tw_R = 0.006328*permeability*(water_mob(n,i))*dy*dz/dx;

Sw(n+1,i) = (Tw_R*(Po(n+1,i+1)-Po(n+1,i)) - Tw_R*(Pcwo(n,i+1)-Pcwo(n,i))...

+ qt)*dt/(VR*porosity)...

- Sw(n,i)*(Cphi+Cw)*(Po(n+1,i)-Po(n,i)) + Sw(n,i);

elseif i == IMAX % Last node

Tt_L = Tt_R;

Tw_L = Tw_R;

Tt_R = 0;

Tw_R = 0;

Sw(n+1,i) = (- Tw_L*(Po(n+1,i)-Po(n+1,i-1)) ...

+ Tw_L*(Pcwo(n,i)-Pcwo(n,i-1))...

- WIw(n,1)*(Po(n+1,i)-Pwell))*dt/(VR*porosity)...

- Sw(n,i)*(Cphi+Cw)*(Po(n+1,i)-Po(n,i)) + Sw(n,i);

else % Intermediate nodes

[krw(n,i),kro(n,i)] = getRelPerm(Sw(n,i),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n,i) = krw(n,i)/water_visc;

oil_mob(n,i) = kro(n,i)/oil_visc;

Tw_L = Tw_R;

Tw_R = 0.006328*permeability*(water_mob(n,i))*dy*dz/dx;

Sw(n+1,i) = (Tw_R*(Po(n+1,i+1)-Po(n+1,i)) - Tw_L*(Po(n+1,i)-Po(n+1,i-1)) ...

- Tw_R*(Pcwo(n,i+1)-Pcwo(n,i)) + Tw_L*(Pcwo(n,i)-Pcwo(n,i-1)))*dt/(VR*porosity)...

- Sw(n,i)*(Cphi+Cw)*(Po(n+1,i)-Po(n,i)) + Sw(n,i);

end

end

Pcwo(n+1,:) = getPcwo(Sw(n+1,:),alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min);

Pw(n+1,:) = Po(n+1,:)-Pcwo(n+1,:);

% Well index

[krw(n+1,IMAX),kro(n+1,IMAX)] = getRelPerm(Sw(n+1,IMAX),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob(n+1,IMAX) = krw(n+1,IMAX)/water_visc;

oil_mob(n+1,IMAX) = kro(n+1,IMAX)/oil_visc;

WI(n+1,1) = 0.006328*permeability*(water_mob(n+1,IMAX)+oil_mob(n+1,IMAX))*dy*dz/(dx/2);

WIw(n+1,1) = WI(n+1,1).*water_mob(n+1,IMAX)./(water_mob(n+1,IMAX)+oil_mob(n+1,IMAX));

end

figure

plot([dx/2:dx:dx*IMAX-dx/2],Sw([1:5:100],:),'linewidth',2)

xlabel('X coordinate (ft)')

ylabel('Water saturation (fraction)')

axis([0,IMAX*dx,0,1])

axis square

qo = WI.*(Po(:,IMAX)-Pwell).*oil_mob(:,IMAX)./(water_mob(:,IMAX)+oil_mob(:,IMAX));

cum_oil = 0;

for i = 2:length(qo)

cum_oil(i,1) = qo(i-1)*dt + cum_oil(i-1);

end

OOIP = sum(VR*porosity*(1-Sw(1,:)));

RecoveryFactor = cum_oil/OOIP*100;

figure

plot([0:dt:tmax],RecoveryFactor(1:101,1),'linewidth',2)

xlabel('Time (days)')

ylabel('Recovery Factor (%)')

Sub-routines

%===== getRelPerm.m

%

% by Luis Zerpa

% Nov 2009

%

function [krw,kro] = getRelPerm(Sw,Sorw,Swr,nw,now,krw_ast,krow_ast)

for i = 1:length(Sw)

if Sw(i) < Swr

krw(i) = 0;

else

krw(i) = krw_ast*((Sw-Swr)/(1-Swr-Sorw)).^nw;

end

if 1-Sw(i) < Sorw

kro(i) = 0;

else

kro(i) = krow_ast*((1-Sw-Sorw)/(1-Swr-Sorw)).^now;

end

end

%==== getPcwo.m

%

% by Luis Zerpa

% Nov 2009

%

function [Pcwo] = getPcwo(Sw,alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min)

for i = 1:length(Sw)

if Sw(i) <= Swr

Pcwo(i) = Pcwo_max;

elseif Sw(i) >= 1-Sorw

Pcwo(i) = Pcwo_min;

elseif Sw(i) < Swx & Sw(i) > Swr

Pcwo2 = -alpha*log((Sw(i)-Swr)/(Swx-Swr));

Pcwo(i) = min(Pcwo2,Pcwo_max);

else

Pcwo1 = alpha*log((1-Sw(i)-Sorw)/(1-Swx-Sorw));

Pcwo(i) = max(Pcwo1,Pcwo_min);

end

end

%===== tdma.m

%

% Thomas Algorithm for linear systems with tri-diagonal matrix

%

%  Luis Zerpa

%  January 2004

%

function [x] = tdma(e,f,g,B)

n = length(B);

% Descomposicion

for k = 2:n

e(k) = e(k)/f(k-1);

f(k) = f(k) - e(k)*g(k-1);

end

% Sustitucion hacia adelante

for k = 2:n

B(k) = B(k) - e(k)*B(k-1);

end

% Sustitucion hacia atras

x(n,1) = B(n)/f(n);

for k = n-1:-1:1

x(k) = (B(k) - g(k)*x(k+1))/f(k);

end

U(1,1) = f(1); U(1,2) = g(1); L(1,1) = 1;

L(n,n-1) = e(n); U(n,n) = f(n); L(n,n) = 1;

for i = 1:n-1

L(i,i) = 1;

L(i+1,i) = e(i+1);

U(i,i) = f(i);

U(i,i+1) = g(i);

end