PEGN 513

Reservoir Simulation I

Fall 2009

Homework # 13

Due Date: 12/01/2009

Luis E. Zerpa

Homework #13

a) Implement project option 4, 1-D formulation for the fully implicit simultaneous approach.

The 1-D fully implicit simultaneous solution of water and oil saturation equation was implemented and coded in a Matlab script, shown in Addendum A.1, using the formulation presented in the HW #12 report.

b) Results of the simulation

Fully implicit formulation results

Figure 1, shows a plot of oil pressure as a function of distance along path of flow at every five time steps, obtained using the fully implicit simultaneous formulation.

Figure 1. Oil pressure as a function of distance along path of flow at every five time steps (Fully implicit formulation).

Figure 2, shows a plot of the water saturation profile as a function of distance along path of flow at every five time steps, obtained using the fully implicit formulation.

Figure 2. Water saturation profile as a function of x coordinate, for every five time steps 
(Fully implicit formulation).

Figure 3, shows a plot of the recovery factor as a function of time, obtained with the fully implicit formulation. The total recovery factor at 10 days is 54.466 % of OOIP.

Figure 3. Recovery factor as a function of time (IMPES formulation).

Results comparison

To compare the results from different formulations, a plot of oil pressure as a function of distance along path of flow at 1.5 days is presented in Figure 4, for the fully implicit, IMPES and Semi-implicit formulations. The results for the IMPES and Semi-implicit formulations are equal, while the solution for the Fully Implicit formulation gives slightly higher pressure values at the initial nodes and match the solution of the other two formulations at the last portion of the porous medium from 60 to 100 ft.

Figure 4. Pressure solution at 1.5 days obtained with the Fully Implicit, IMPES and Semi-implicit formulations.

The water saturation profiles at 1.5 days obtained with the Fully implicit, IMPES and Semi-implicit formulations is presented in Figure 5. The fully implicit solution looks more spread due to more numerical dispersion.

Figure 5. Water saturation profile at 1.5 days obtained with the Fully implicit, IMPES and 
Semi-implicit formulations.

Between the Fully Implicit, IMPES and Semi-implicit formulations, the one with the greatest recovery factor at the end of the simulation (10 days) is the Fully Implicit with 54.466 % of OOIP. The recovery factor obtained with the IMPES formulation is equal to that of the Semi-implicit formulation (51.45 %).

The water saturation profiles at 1.5 days obtained with the Fully Implicit, IMPES and Semi-implicit formulations and with the four cases studied in HW #3 (Buckley-Leverett) are shown in Figure 6. The case that shows a greater numerical dispersion, manifested by a spreader saturation curve is HW#3 Case 3, the implicit Buckley-Leverett solution with coarse grid. The explicit Buckley-Leverett Case 2 with finer grid represents the case with the lowest numerical dispersion, showing a sharper water front. The solution obtained with the numerical cases based on the solution of the pressure and saturation equation using IMPES and semi-implicit formulations are equal. The solution obtained with the Fully Implicit formulation has lower water saturations at initial portion of the porous medium, showing a disperse water front.

Figure 6. Water saturation profile at 1.5 days obtained with the Fully implicit, IMPES and
Semi-implicit formulations with the four Buckley-Leveret cases studied in HW#3.

Addendum A.1 - MATLAB script for Fully Implicit formulation

%===== PEGN513_HW13.m

% 1D Fully implicit solution of the water and oil Saturation equations

%

% Luis Zerpa

% 25 Nov 2009

clc,clear all,close all

dt = 0.1; % days

tmax = 10; % days

nTimeSteps = tmax/dt+1;

% GRID

IMAX = 10; % ft

dx = 10; % ft

dy = 10; % ft

dz = 10; % ft

VR = dx*dy*dz; D = zeros(1,IMAX);

% ROCK PROPERTIES

krw_ast = 0.1; krow_ast = 0.7; nw = 1.5; now = 2.5; Sorw = 0.3; Swr = 0.25;

alpha = 0.8; Swx = 0.5; Pcwo_max = 5; Pcwo_min = -5;

porosity = 0.2; permeability = 100; % md

% FLUID PROPERTIES

water_visc = 0.6; % cp

oil_visc = 2.4; % cp

gamma_w = 0.433; % psi/ft

gamma_o = 0.433*0.8; % psi/ft

% INJECTION RATE

qt = 200; % ft^3/day

% INITIAL AND BOUNDARY CONDITIONS

Pi = 2000; % psia

Pwell = 1900; % psia

Swi = 0.25;

% COMPRESSIBILITIES

Cphi = 3e-6; % 1/psi

Cw = 4e-6; % 1/psi

Co = 10e-6; % 1/psi

% INITIALIZATION

Po = Pi*ones(1,IMAX);

Sw = Swi*ones(1,IMAX);

So = 1-Sw;

% CAPILLARY PRESSURE

Pcwo = getPcwo(Sw,alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min);

Pw = Po-Pcwo;

% WELL INDEX

[krw,kro] = getRelPerm(Sw(1,IMAX),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob = krw/water_visc;

oil_mob = kro/oil_visc;

WIw = 0.006328*permeability*water_mob*dy*dz/(dx/2);

WIo = 0.006328*permeability*oil_mob*dy*dz/(dx/2);

% PRODUCTION RATES

qw = -WIw*(Po(IMAX)-Pwell);

qo = -WIo*(Po(IMAX)-Pwell);

% INITIALIZE FIRST ITERATION

Po_l = Po;

Sw_l = Sw;

Pcwo_l = Pcwo;

WIw_l = WIw;

WIo_l = WIo;

qw_l = qw;

qo_l = qo;

for n = 1:nTimeSteps

deltas_vector = 50*ones(2*IMAX,1);

deltas_vector_old = ones(2*IMAX,1);

while max(deltas_vector) > 1e-4

% Build coefficient matrix

%==== Initialization of matrix

A = zeros(IMAX*2,IMAX*2);

dPcwo_dSw = getdPcwo_dSw(Sw_l,alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min);

[krw,kro] = getRelPerm(Sw_l,Sorw,Swr,nw,now,krw_ast,krow_ast);

dKrw_dSw = getdKr_dSw(Sw_l,Sorw,Swr,nw,now,krw_ast,krow_ast,'water');

dKro_dSw = getdKr_dSw(Sw_l,Sorw,Swr,nw,now,krw_ast,krow_ast,'oil');

for i = 1: IMAX

if i == 1 % First node

%=============== WATER =================

Tw_L = 0;

% Flow potential WATER

FlowPotentialw_i = Po_l(i) - gamma_w*D(i);

FlowPotentialw_iR = Po_l(i+1) - gamma_w*D(i+1);

if FlowPotentialw_i >= FlowPotentialw_iR

water_mob = krw(i)/water_visc;

Tw_R = 0.006328*permeability*water_mob*dy*dz/dx;

dTw_dSw_R = 0.006328*permeability*dKrw_dSw(i)*dy*dz/(dx*water_visc);

else

water_mob = krw(i+1)/water_visc;

Tw_R = 0.006328*permeability*water_mob*dy*dz/dx;

dTw_dSw_R = 0.006328*permeability*dKrw_dSw(i+1)*dy*dz/(dx*water_visc);

end

% initiallize A_template

A_template = zeros(2,6);

% W_deltaPoi

A_template(1,3) = -(Tw_L + Tw_R + VR*porosity*Sw_l(i)*(Cphi + Cw)/dt);

% W_deltaPoi+1

A_template(1,5) = Tw_R;

% W_deltaSwi

A_template(1,4) = Tw_R*dPcwo_dSw(i) + Tw_L*dPcwo_dSw(i) - VR*porosity/dt;

% W_deltaSwi+1

A_template(1,6) = -Tw_R*dPcwo_dSw(i+1);

% W_deltaSwi+1/2

if FlowPotentialw_i >= FlowPotentialw_iR

A_template(1,4) = A_template(1,4) + dTw_dSw_R*(Po_l(i+1)-Po_l(i)) - gamma_w*(D(i+1)-D(i))*dTw_dSw_R-dTw_dSw_R*(Pcwo_l(i+1)-Pcwo_l(i));

else

A_template(1,6) = A_template(1,6) + dTw_dSw_R*(Po_l(i+1)-Po_l(i)) - gamma_w*(D(i+1)-D(i))*dTw_dSw_R -dTw_dSw_R*(Pcwo_l(i+1)-Pcwo_l(i));

end

%=============== OIL =================

To_L = 0;

% Flow potential OIL

FlowPotentialo_i = Po_l(i) - gamma_o*D(i);

FlowPotentialo_iR = Po_l(i+1) - gamma_o*D(i+1);

if FlowPotentialo_i >= FlowPotentialo_iR

oil_mob= kro(i)/oil_visc;

To_R = 0.006328*permeability*oil_mob*dy*dz/dx;

dTo_dSw_R = 0.006328*permeability*dKro_dSw(i)*dy*dz/(dx*oil_visc);

else

oil_mob = kro(i+1)/oil_visc;

To_R = 0.006328*permeability*oil_mob*dy*dz/dx;

dTo_dSw_R = 0.006328*permeability*dKro_dSw(i+1)*dy*dz/(dx*oil_visc);

end

% O_deltaPoi

A_template(2,3) = -(To_L + To_R + VR*porosity*(1-Sw_l(i))*(Cphi + Co)/dt);

% O_deltaPoi+1

A_template(2,5) = To_R;

% O_deltaSwi

A_template(2,4) = VR*porosity/dt; % dSwi

% O_deltaSwi+1

A_template(2,6) = 0;

% O_deltaSwi+1/2

if FlowPotentialo_i >= FlowPotentialo_iR

A_template(2,4) = A_template(2,4) + dTo_dSw_R*(Po_l(i+1)-Po_l(i)) - gamma_o*(D(i+1)-D(i))*dTo_dSw_R;

else

A_template(2,6) = A_template(2,6) + dTo_dSw_R*(Po_l(i+1)-Po_l(i)) - gamma_o*(D(i+1)-D(i))*dTo_dSw_R;

end

% W_residual

R(2*(i-1)+1,1) = VR*porosity*(Sw_l(i)-Sw(n,i))/dt – qt - Tw_R*(Po_l(i+1)-Po_l(i)) + Tw_R*gamma_w*(D(i+1)-D(i)) + Tw_R*(Pcwo_l(i+1)-Pcwo_l(i));

% O_residual

R(2*(i-1)+2,1) = VR*porosity*(Sw(n,i)-Sw_l(i))/dt - To_R*(Po_l(i+1)-Po_l(i)) + To_R*gamma_o*(D(i+1)-D(i));

A(1:2,1:4) = A_template(:,3:6);

elseif i == IMAX % Last node

%=============== WATER =================

% Flow potential WATER

FlowPotentialw_iL = Po_l(i-1) - gamma_w*D(i-1);

FlowPotentialw_i = Po_l(i) - gamma_w*D(i);

% Left interface

if FlowPotentialw_i >= FlowPotentialw_iL

water_mob = krw(i)/water_visc;

Tw_L = 0.006328*permeability*water_mob*dy*dz/dx;

dTw_dSw_L = 0.006328*permeability*dKrw_dSw(i)*dy*dz/(dx*water_visc);

else

water_mob = krw(i-1)/water_visc;

Tw_L = 0.006328*permeability*water_mob*dy*dz/dx;

dTw_dSw_L = 0.006328*permeability*dKrw_dSw(i-1)*dy*dz/(dx*water_visc);

end

% Right interface

Tw_R = 0;

dTw_dSw_R = 0;

% initiallize A_template

A_template = zeros(2,6);

% W_deltaPoi-1

A_template(1,1) = Tw_L;

% W_deltaPoi

A_template(1,3) = -(Tw_L + Tw_R + VR*porosity*Sw_l(i)*(Cphi + Cw)/dt + WIw_l);

% W_deltaPoi+1

A_template(1,5) = Tw_R;

% W_deltaSwi-1

A_template(1,2) = -Tw_L*dPcwo_dSw(i-1);

% W_deltaSwi

A_template(1,4) = (Tw_R + Tw_L)*dPcwo_dSw(i) - VR*porosity/dt...

-0.006328*permeability*(dKrw_dSw(i)/water_visc)*(dy*dz/(dx/2))*(Po_l(i)-Pwell);

% W_deltaSwi+1

A_template(1,6) = 0;

% W_deltaSwi-1/2

if FlowPotentialw_i >= FlowPotentialw_iL

A_template(1,4) = A_template(1,4) - dTw_dSw_L*(Po_l(i)-Po_l(i-1)) + gamma_w*(D(i)-D(i-1))*dTw_dSw_L...

+ dTw_dSw_L*(Pcwo_l(i)-Pcwo_l(i-1));

else

A_template(1,2) = A_template(1,2) - dTw_dSw_L*(Po_l(i)-Po_l(i-1)) + gamma_w*(D(i)-D(i-1))*dTw_dSw_L...

+ dTw_dSw_L*(Pcwo_l(i)-Pcwo_l(i-1));

end

%=============== OIL =================

% Flow potential OIL

FlowPotentialo_iL = Po_l(i-1) - gamma_o*D(i-1);

FlowPotentialo_i = Po_l(i) - gamma_o*D(i);

% Left interface

if FlowPotentialo_i >= FlowPotentialo_iL

oil_mob= kro(i)/oil_visc;

To_L = 0.006328*permeability*oil_mob*dy*dz/dx;

dTo_dSw_L = 0.006328*permeability*dKro_dSw(i)*dy*dz/(dx*oil_visc);

else

oil_mob = kro(i-1)/oil_visc;

To_L = 0.006328*permeability*oil_mob*dy*dz/dx;

dTo_dSw_L = 0.006328*permeability*dKro_dSw(i-1)*dy*dz/(dx*oil_visc);

end

% Right interface

To_R = 0; dTo_dSw_R = 0;

% O_deltaPoi-1

A_template(2,1) = To_L;

% O_deltaPoi

A_template(2,3) = -(To_L + To_R + VR*porosity*(1-Sw_l(i))*(Cphi + Co)/dt + WIo_l);

% O_deltaPoi+1

A_template(2,5) = To_R;

% O_deltaSwi-1

A_template(2,2) = 0;

% O_deltaSwi

A_template(2,4) = VR*porosity/dt...

- 0.006328*permeability*(dKro_dSw(i)/oil_visc)*(dy*dz/(dx/2))*(Po_l(i)-Pwell);

% O_deltaSwi+1

A_template(2,6) = 0;

% O_deltaSwi-1/2

if FlowPotentialo_i >= FlowPotentialo_iL

A_template(2,4) = A_template(2,4) - dTo_dSw_L*(Po_l(i)-Po_l(i-1)) + gamma_o*(D(i)-D(i-1))*dTo_dSw_L;

else

A_template(2,2) = A_template(2,2) - dTo_dSw_L*(Po_l(i)-Po_l(i-1)) + gamma_o*(D(i)-D(i-1))*dTo_dSw_L;

end

% W_residual

R(2*(i-1)+1,1) = VR*porosity*(Sw_l(i)-Sw(n,i))/dt...

-qw_l + Tw_L*(Po_l(i)-Po_l(i-1)) - Tw_L*gamma_w*(D(i)-D(i-1)) - Tw_L*(Pcwo_l(i)-Pcwo_l(i-1));

% O_residual

R(2*(i-1)+2,1) = VR*porosity*(Sw(n,i)-Sw_l(i))/dt ...

- qo_l + To_L*(Po_l(i)-Po_l(i-1)) - To_L*gamma_o*(D(i)-D(i-1));

A(2*(IMAX-1)+1:2*(IMAX-1)+2,2*(IMAX-2)+1:2*(IMAX-2)+4) = A_template(:,1:4);

else % Intermediate nodes

%=============== WATER =================

% Flow potential WATER

FlowPotentialw_iL = Po_l(i-1) - gamma_w*D(i-1);

FlowPotentialw_i = Po_l(i) - gamma_w*D(i);

FlowPotentialw_iR = Po_l(i+1) - gamma_w*D(i+1);

% Left interface

if FlowPotentialw_i >= FlowPotentialw_iL

water_mob = krw(i)/water_visc;

Tw_L = 0.006328*permeability*water_mob*dy*dz/dx;

dTw_dSw_L = 0.006328*permeability*dKrw_dSw(i)*dy*dz/(dx*water_visc);

else

water_mob = krw(i-1)/water_visc;

Tw_L = 0.006328*permeability*water_mob*dy*dz/dx;

dTw_dSw_L = 0.006328*permeability*dKrw_dSw(i-1)*dy*dz/(dx*water_visc);

end

% Right interface

if FlowPotentialw_i >= FlowPotentialw_iR

water_mob = krw(i)/water_visc;

Tw_R = 0.006328*permeability*water_mob*dy*dz/dx;

dTw_dSw_R = 0.006328*permeability*dKrw_dSw(i)*dy*dz/(dx*water_visc);

else

water_mob = krw(i+1)/water_visc;

Tw_R = 0.006328*permeability*water_mob*dy*dz/dx;

dTw_dSw_R = 0.006328*permeability*dKrw_dSw(i+1)*dy*dz/(dx*water_visc);

end

% initiallize A_template

A_template = zeros(2,6);

% W_deltaPoi-1

A_template(1,1) = Tw_L;

% W_deltaPoi

A_template(1,3) = -(Tw_L + Tw_R + VR*porosity*Sw_l(i)*(Cphi + Cw)/dt);

% W_deltaPoi+1

A_template(1,5) = Tw_R;

% W_deltaSwi-1

A_template(1,2) = -Tw_L*dPcwo_dSw(i-1);

% W_deltaSwi

A_template(1,4) = (Tw_R + Tw_L)*dPcwo_dSw(i) - VR*porosity/dt;

% W_deltaSwi+1

A_template(1,6) = -Tw_R*dPcwo_dSw(i+1);

% W_deltaSwi-1/2

if FlowPotentialw_i >= FlowPotentialw_iL

A_template(1,4) = A_template(1,4) - dTw_dSw_L*(Po_l(i)-Po_l(i-1)) + gamma_w*(D(i)-D(i-1))*dTw_dSw_L...

+ dTw_dSw_L*(Pcwo_l(i)-Pcwo_l(i-1));

else

A_template(1,2) = A_template(1,2) - dTw_dSw_L*(Po_l(i)-Po_l(i-1)) + gamma_w*(D(i)-D(i-1))*dTw_dSw_L...

+ dTw_dSw_L*(Pcwo_l(i)-Pcwo_l(i-1));

end

% W_deltaSwi+1/2

if FlowPotentialw_i >= FlowPotentialw_iR

A_template(1,4) = A_template(1,4) + dTw_dSw_R*(Po_l(i+1)-Po_l(i)) - gamma_w*(D(i+1)-D(i))*dTw_dSw_R...

- dTw_dSw_R*(Pcwo_l(i+1)-Pcwo_l(i));

else

A_template(1,6) = A_template(1,6) + dTw_dSw_R*(Po_l(i+1)-Po_l(i)) - gamma_w*(D(i+1)-D(i))*dTw_dSw_R...

- dTw_dSw_R*(Pcwo_l(i+1)-Pcwo_l(i));

end

%=============== OIL =================

% Flow potential OIL

FlowPotentialo_iL = Po_l(i-1) - gamma_o*D(i-1);

FlowPotentialo_i = Po_l(i) - gamma_o*D(i);

FlowPotentialo_iR = Po_l(i+1) - gamma_o*D(i+1);

% Left interface

if FlowPotentialo_i >= FlowPotentialo_iL

oil_mob= kro(i)/oil_visc;

To_L = 0.006328*permeability*oil_mob*dy*dz/dx;

dTo_dSw_L = 0.006328*permeability*dKro_dSw(i)*dy*dz/(dx*oil_visc);

else

oil_mob = kro(i-1)/oil_visc;

To_L = 0.006328*permeability*oil_mob*dy*dz/dx;

dTo_dSw_L = 0.006328*permeability*dKro_dSw(i-1)*dy*dz/(dx*oil_visc);

end

% Right interface

if FlowPotentialo_i >= FlowPotentialo_iR

oil_mob= kro(i)/oil_visc;

To_R = 0.006328*permeability*oil_mob*dy*dz/dx;

dTo_dSw_R = 0.006328*permeability*dKro_dSw(i)*dy*dz/(dx*oil_visc);

else

oil_mob = kro(i+1)/oil_visc;

To_R = 0.006328*permeability*oil_mob*dy*dz/dx;

dTo_dSw_R = 0.006328*permeability*dKro_dSw(i+1)*dy*dz/(dx*oil_visc);

end

% O_deltaPoi-1

A_template(2,1) = To_L;

% O_deltaPoi

A_template(2,3) = -(To_L + To_R + VR*porosity*(1-Sw_l(i))*(Cphi + Co)/dt);

% O_deltaPoi+1

A_template(2,5) = To_R;

% O_deltaSwi-1

A_template(2,2) = 0;

% O_deltaSwi

A_template(2,4) = VR*porosity/dt; % dSwi

% O_deltaSwi+1

A_template(2,6) = 0;

% O_deltaSwi-1/2

if FlowPotentialo_i >= FlowPotentialo_iL

A_template(2,4) = A_template(2,4) - dTo_dSw_L*(Po_l(i)-Po_l(i-1)) + gamma_o*(D(i)-D(i-1))*dTo_dSw_L;

else

A_template(2,2) = A_template(2,2) - dTo_dSw_L*(Po_l(i)-Po_l(i-1)) + gamma_o*(D(i)-D(i-1))*dTo_dSw_L;

end

% O_deltaSwi+1/2

if FlowPotentialo_i >= FlowPotentialo_iR

A_template(2,4) = A_template(2,4) + dTo_dSw_R*(Po_l(i+1)-Po_l(i)) - gamma_o*(D(i+1)-D(i))*dTo_dSw_R;

else

A_template(2,6) = A_template(2,6) + dTo_dSw_R*(Po_l(i+1)-Po_l(i)) - gamma_o*(D(i+1)-D(i))*dTo_dSw_R;

end

% W_residual

R(2*(i-1)+1,1) = VR*porosity*(Sw_l(i)-Sw(n,i))/dt...

- Tw_R*(Po_l(i+1)-Po_l(i)) + Tw_L*(Po_l(i)-Po_l(i-1))...

+ Tw_R*gamma_w*(D(i+1)-D(i)) - Tw_L*gamma_w*(D(i)-D(i-1)) ...

+ Tw_R*(Pcwo_l(i+1)-Pcwo_l(i)) - Tw_L*(Pcwo_l(i)-Pcwo_l(i-1));

% O_residual

R(2*(i-1)+2,1) = VR*porosity*(Sw(n,i) - Sw_l(i))/dt ...

- To_R*(Po_l(i+1)-Po_l(i)) + To_L*(Po_l(i)-Po_l(i-1))...

+ To_R*gamma_o*(D(i+1)-D(i)) - To_L*gamma_o*(D(i)-D(i-1));

A(2*(i-1)+1:2*(i-1)+2,2*(i-2)+1:2*(i-2)+6) = A_template(:,1:6);

end

end

% Solve for delta_Po and delta_Sw

deltas_vector = A\R;

% Update variables for new iteration l+1

Po_l = Po_l + deltas_vector(1:2:2*IMAX)';

Sw_l = Sw_l + deltas_vector(2:2:2*IMAX)';

Pcwo_l = getPcwo(Sw_l,alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min);

% Well index

[krw,kro] = getRelPerm(Sw_l(IMAX),Sorw,Swr,nw,now,krw_ast,krow_ast);

water_mob = krw/water_visc;

oil_mob = kro/oil_visc;

WIw_l = 0.006328*permeability*water_mob*dy*dz/(dx/2);

WIo_l = 0.006328*permeability*oil_mob*dy*dz/(dx/2);

qw_l = -WIw_l*(Po_l(IMAX)-Pwell);

qo_l = -WIo_l*(Po_l(IMAX)-Pwell);

end % While loop

Po(n+1,:) = Po_l;

Sw(n+1,:) = Sw_l;

Pcwo(n+1,:) = Pcwo_l;

WIw(n+1,1) = WIw_l;

WIo(n+1,1) = WIo_l;

qw(n+1,1) = qw_l;

qo(n+1,1) = qo_l;

end

figure

plot([dx/2:dx:dx*IMAX-dx/2],Po([1:5:100],:),'linewidth',2)

xlabel('X coordinate (ft)')

ylabel('Oil Pressure (psi)')

figure

plot([dx/2:dx:dx*IMAX-dx/2],Sw([1:5:100],:),'linewidth',2)

xlabel('X coordinate (ft)')

ylabel('Water saturation (fraction)')

axis([0,IMAX*dx,0,1])

axis square

cum_oil = 0;

for i = 2:length(qo)

cum_oil(i,1) = -qo(i-1)*dt + cum_oil(i-1);

end

OOIP = sum(VR*porosity*(1-Sw(1,:)));

RecoveryFactor = cum_oil/OOIP*100;

figure

plot([0:dt:tmax],RecoveryFactor(1:101,1),'linewidth',2)

xlabel('Time (days)')

ylabel('Recovery Factor (%)')

Sub-routines

%===== getRelPerm.m

%

% by Luis Zerpa

% Nov 2009

%

function [krw,kro] = getRelPerm(Sw,Sorw,Swr,nw,now,krw_ast,krow_ast)

for i = 1:length(Sw)

if Sw(i) < Swr

krw(i) = 0;

else

krw(i) = krw_ast*((Sw-Swr)/(1-Swr-Sorw)).^nw;

end

if 1-Sw(i) < Sorw

kro(i) = 0;

else

kro(i) = krow_ast*((1-Sw-Sorw)/(1-Swr-Sorw)).^now;

end

end

%==== getPcwo.m

%

% by Luis Zerpa

% Nov 2009

%

function [Pcwo] = getPcwo(Sw,alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min)

for i = 1:length(Sw)

if Sw(i) <= Swr

Pcwo(i) = Pcwo_max;

elseif Sw(i) >= 1-Sorw

Pcwo(i) = Pcwo_min;

elseif Sw(i) < Swx & Sw(i) > Swr

Pcwo2 = -alpha*log((Sw(i)-Swr)/(Swx-Swr));

Pcwo(i) = min(Pcwo2,Pcwo_max);

else

Pcwo1 = alpha*log((1-Sw(i)-Sorw)/(1-Swx-Sorw));

Pcwo(i) = max(Pcwo1,Pcwo_min);

end

end

%==== getdPcwo_dSw.m

%

% Derivative of Water-Oil Capillary pressure with respect to water saturation

%

% Luis Zerpa

% 15 Nov 2009

%

function [dPcwo_dSw] = getdPcwo_dSw(Sw,alpha,Sorw,Swr,Swx,Pcwo_max,Pcwo_min)

for i = 1:length(Sw)

if Sw(i) <= Swr

dPcwo_dSw(i) = 0;

elseif Sw(i) >= 1-Sorw

dPcwo_dSw(i) = 0;

elseif Sw(i) < Swx & Sw(i) > Swr

dPcwo_dSw(i) = -alpha/(Sw(i)-Swr);

else

dPcwo_dSw(i) = -alpha/(1-Sw(i)-Sorw);

end

end

%===== getdKr_dSw.m

function [dKr_dSw] = getdKr_dSw(Sw,Sorw,Swr,nw,now,krw_ast,krow_ast,phase)

switch phase

case 'water'

for i = 1:length(Sw)

if Sw(i) < Swr

dKr_dSw(i) = 0;

else

dKr_dSw(i) = krw_ast*nw*((Sw(i)-Swr)/(1-Swr-Sorw)).^(nw-1)*((1)/(1-Swr-Sorw));

end

end

case 'oil'

for i = 1:length(Sw)

if 1-Sw(i) < Sorw

dKr_dSw(i) = 0;

else

dKr_dSw(i) = krow_ast*now*((1-Sw(i)-Sorw)/(1-Swr-Sorw)).^(now-1)*((-1)/(1-Swr-Sorw));

end

end

end