HW#19

Assigned: Wednesday, March 25, 2009

Due: Thursday, April 2, 2009 in class

Please answer question #2 from exam #2.

#2:

A common field steam generator is a 50 mega Btu/hr unit. In generating steam in one of these units, the feed water and discharged steam had the following characteristics:

Feed Water:

Pressure		1,300 psia

Temperature	70

Enthalpy		41.65 But/lbm

Flow rate		48,960 lbm/hr

Steam Discharge:

Pressure		1000 psia

Temperature	544

Enthalpy		1063 Btu/lbm

Quality		80%

Flow rate		48,579 lbm/hr

- For a barrel of water converted to 80% quality wet steam, how much natural gas is required? Assume 1,000,000 Btu/Mscf, and boiler efficiency of 85%.

- If we use one BOE of natural gas to generate steam as in Part (a), how many barrels of water will be converted to steam for each BOE of natural gas?

- If the average oil-steam-ratio is 0.20, how many barrels of oil are produced per BOE of natural gas consumed?