HW#2

Assignment: January 15, 2009 						Due: January 20, 2009

How many tons of coal will be used to generate electricity to keep a 100-Watt light bulb for one year?

Given Data:

- 1 kW= 3,414 Btuhr

- Coal provides energy of  7,000 Btulbm

- Conversion efficiency of coal power plant to electricity to transmission, etc. = 35%